| Major Electrical components List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Item | Part Name | Manufacturers | Model Number | Certified Information |
| 01 | SW1 | Switch Disconnector | Moeller | P1-25/EA/SVB | K005797 E36332 |
| 02 | SB01 | Emergency Stop | Schneider | ZB4 | MFR Doc |
| 03 | F3 | Fuse | Jenn Feng Electric Industrial Co.,Ltd., | MFA 250V AC 3A To protect T01 | --- |
| 04 | F5 | Fuse | Jenn Feng Electric Industrial Co.,Ltd., | MFA 250V AC 5A To protect T01, T03, M4 and M5 | --- |
| 05 | F10 | Fuse | Jenn Feng Electric Industrial Co.,Ltd., | MMG52 250V AC 10A To protect T04 and T05 | --- |
| 06 | F30 | Fuse | ---- | 600V 30A | --- |
| 07 | C1 | Switch Contactor | Moeller | DIL0A M 220/380V (7-1/2Hp or 35 A) | K 03697 |
| 08 | C2 | Switch Contactor | Moeller | DIL00A M-01 220/380V (3 Hp or 20A) | K 03697 |
| 09 | C3 | Switch Contactorr | Moeller | DIL00A M-01 220/380V (3 Hp or 20A) | K 03697 |
| 10 | C4 | Switch Contactor | Moeller | DIL00A M-01 220/380V (3 Hp or 20A) | K 03697 |
| 11 | M 1 | Starting with breaker Motor | KY | KN-202 5Hp 3ψ 4P 50/60 Hz 220/380V 13.5/7.5A 1430/1720 rpm Brake: Coil D.C 20 ~ 24 V 32A | --- |
| 12 | M 2 | Tip Motor | Tong Yeng Electric | 3ψ1/4 Hp 4 P 220/380V 1.0/0.6A 50/60Hz 1430/1720 rpm | --- |
| 13 | M 3 | Heating Element Elevating Motor | Tung Lee Electrical Co., Ltd | 51K60A-S 3ψ 4 P 60W 220/380V 0.4/0.28A 50/60Hz 1300/1550 rpm | AE9839095 01 |
| 14 | M4 | AC Servomotor for Warp Let-Off | Panasonic Matsushita Electric Industrial Co., Ltd. | MSMA042A1E Input: 200~230V 3ψ 1.8A 50/60 Hz Output: 106V 2.5A 0~333.3Hz 400W 3000 rpm | TUV R9950348 CC9950406 |
| 15 | M5 | Wind Up Device AC Servermotor | Panasonic Matsushita Electric Industrial Co., Ltd. | MSMA082A1E Input: 200~230V 3ψ ??A 50/60 Hz Output: 116V 4.3A 200Hz 750W 3000 rpm | TUV R9950348 CC9950406 |
| 16 | TH1 | Starting with breaker Motor Overload Relay | Moeller | Z00-16 220/380V 16A/10A (10 ~ 16A) To protect M1 | K 029895 |
| 17 | TH2 | Tip Motor Overload Relay | Moeller | Z00-1.6 220/380V 1.6A/1.0A (1.0~ 1.6A) To protect M2 | K 029895 |
| 18 | TH3 | Heating Element Elevating Motor Overload Relay | Moeller | Z00-1 220/380V 1.0A/0.6A (0.6 ~ 1.0A) To protect M3 | K 029895 |

| Major Electrical components List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Item | Part Name | Manufacturers | Model Number | Certified Information |
| 19 | S2 | 220/380V Transformer 4KVA Protector | Moeller | FAZ-3-B6 6A (400V) To protect T02 | K 003299 |
| 20 | S3 | Power Supply Transformer Protector | Moeller | FAZ-2-S16 16A (400V) To protect T04, T05 | K 003299 |
| 21 | S4 | Power Supply Transformer Protector | Moeller | FAZ-2-S16 16A (400V) | K 003299 |
| 22 | LS1 | Limit Switch | Omron | D4D1532 for weaving cover | R9451184 |
| 23 | T01 | 220/380V Transformer | --- | Input: 380V/ 220V 50/60Hz 7 A 50/60Hz Output: 18V, 24V | --- |
| 24 | T02 | 220/380V 4KVA Transformer Used only for Machine rated more than 380V. | --- | Input: 380V 50/60Hz Output: 220V 50/60Hz 4KVA | --- |
| 25 | T03 | Transformer | --- | Input： 380V/30A Output：200~230V | --- |
| 26 | T04 | Transformer | --- | Input： 380V/5A Output：20V | --- |
| 27 | T05 | Transformer | --- | Input： 380V/60A Output：200~230V | --- |
| 28 |  | Temperature Control | --- | 150± 5 ℃ | --- |
| 29 | SSR |  | Fotek Sensor Co., Ltd., | SSR-25DA, 25A, 24V 380V AC | TUV CC9653517 |
| 30 |  | Heating Element | --- | 1200W | --- |
| 31 |  | Sensor | --- | PS-10N | --- |